# FRC Climber Subsystem Documentation (Climber.java)

## Overview

This document describes the design and operation of the `Climber` subsystem used in an FRC robot. The subsystem provides motor control, soft-limit protection, a bottom limit switch input, encoder-based telemetry, and several command factories for manual and semi-automatic climbing behaviors.

## File & Key Dependencies

• File: frc.robot.subsystems.Climber
• Depends on: `frc.robot.Constants.ClimberConstants` for ports and configuration.
• Libraries: WPILib (Commands, DigitalInput, SmartDashboard), REVLib (SparkMax, RelativeEncoder).

## Hardware & I/O

• Motor: One REV Spark Max (brushless) controlling a climber motor.
• Encoder: `RelativeEncoder` from the Spark Max for position feedback.
• Limit Switch: `DigitalInput` used to detect the mechanical limit (typically bottom).

## Motor & Safety Configuration

• Idle Mode: Brake — resists motion when no power is applied to hold position.
• Soft Limits: Configured via `SparkMaxConfig`:
    – Reverse (downward) soft limit: −390 (enabled)
    – Forward (upward) soft limit: +1 (enabled)
  These prevent the motor from moving beyond the configured encoder positions.
• Encoder: Read from the integrated motor encoder; no conversion factors are set in this class.
• Persistence: Config applied with `ResetMode.kResetSafeParameters` and `PersistMode.kPersistParameters` to survive power cycles.

## Telemetry & Diagnostics

The `periodic()` method publishes the following keys to SmartDashboard:
• `climb limit` — Boolean state of the digital limit switch (true/false).
• `climber position` — Current encoder position (units are raw rotations/ticks based on default REV settings).

## Command Factories

All commands are created using `FunctionalCommand` and share the `Climber` subsystem requirement.

1) `Stop()`
   • Behavior: Immediately sets motor output to 0 in init and execute; never finishes on its own.
   • Use Case: Establish a default/idle command or an emergency stop.

2) `OnClimb(DoubleSupplier speed)`
   • Behavior: Drives motor at +speed continuously; stops on end/interruption.
   • Use Case: Manual climb in the forward (up) direction (sign convention depends on wiring/encoder).

3) `OffClimb(DoubleSupplier speed)`
   • Behavior: Drives motor at −speed continuously; stops on end/interruption.
   • Use Case: Manual climb in the reverse (down) direction.

4) `manualClimber(DoubleSupplier inSpeed, DoubleSupplier outSpeed)`
   • Behavior: Sets output to (inSpeed − outSpeed) allowing two inputs (e.g., RT/LT) to combine.
   • Use Case: Driver/Operator control with incremental in/out adjustments.

5) `fullForward(DoubleSupplier inSpeed)`
   • Behavior: Outputs `inSpeed` in `initialize()`. During `execute()`, logic adjusts output based on encoder position:
       – If position < 80: set `inSpeed`
       – Else if position < 128: set `inSpeed`
       – Else if position < 160: set `inSpeed`
       – Else if position > 160: set `inSpeed * 3`
   • Note: The last branch multiplies the input by 3 beyond 160 units, increasing speed significantly. Ensure this is intentional and mechanically safe, and that soft limits remain effective for protection.

## Limit Switch Behavior

Method: `getLimitSwitch()` returns the raw digital input. True/false semantics depend on wiring (NO/NC). The class does not automatically zero the encoder or stop the motor based on this switch; that logic should be handled by commands or higher-level state machines as needed.

## Recommended Usage Patterns

• Default Command: Consider `Stop()` as the default to hold position safely (with brake mode).
• Manual Mode: Bind operator controls to `manualClimber(in, out)` for intuitive control.
• Directional Buttons: Map individual buttons to `OnClimb(speed)` and `OffClimb(speed)` as needed.
• Safety: Enforce checks (limit switch, soft limits, match state) in higher-level commands to prevent misuse.

## Integration Notes

• Constants: All ports/IDs come from `ClimberConstants` (not shown here). Ensure they match wiring.
• Encoder Zeroing: This class does not zero the encoder. Add a homing routine if absolute reference is required.
• Unit Conversions: If real-world units are needed (e.g., millimeters of travel), set encoder conversion factors in the Spark Max configuration.

## Troubleshooting & Tuning

• If the climber moves in the wrong direction, invert motor or swap positive/negative inputs in the commands.
• If soft limits are not working, verify encoder position sign conventions and that soft limits are enabled after any firmware/library updates.
• Use SmartDashboard readouts to monitor `climber position` and `climb limit` during testing.

## Safety Considerations

• Always test at low speed with the robot disabled on blocks before full-speed trials.
• Validate that limit switch wiring and soft limits physically prevent over-travel.
• Review the `fullForward` multiplier beyond 160 encoder units for mechanical safety.