# FRC EndEffector Subsystem Documentation (EndEffector.java)

## Overview

This document describes the `EndEffector` subsystem used to intake, hold, and eject game pieces. It is implemented as a WPILib `SubsystemBase` with command factories built on `FunctionalCommand`. The mechanism uses two REV Spark Max brushless motor controllers (left master, right follower) and two beam-break sensors to manage material flow and avoid jams.

## File & Key Dependencies

• File: frc.robot.subsystems.EndEffector
• Constants: `frc.robot.Constants.IntakeConstants` (CAN IDs, DIO pins, current limit)
• Libraries: WPILib (Commands, Shuffleboard, DigitalInput), REVLib (SparkMax, config), CTRE Phoenix 6 (TalonFX — declared but unused)

## Hardware & I/O

• Motors: Two Spark Max brushless controllers
    – LEFT_SPARKMAX: master (power set via `setIntakePower`)
    – RIGHT_SPARKMAX: follower (`follow(LEFT_SPARKMAX, true)`) with inversion enabled
• Sensors: Two beam-break (DIO) sensors
    – `BBIntake` (forward/entry)
    – `BBDis` (back/exit)
• Dashboard: Shuffleboard tab `EndEffector` shows intake and discharge beam states and a NumberSlider for `Intake Speed`

## Motor Configuration

• Idle Mode: Brake on both Spark Max controllers
• Smart Current Limit: `IntakeConstants.INTAKE_CURRENT_LIMITS`
• Follower Setup: Right Spark Max follows the left with inversion set to `true`
• Persistence: `ResetMode.kResetSafeParameters` and `PersistMode.kPersistParameters` applied for both controllers

## Control Logic

• Power Application: All power is applied through the left Spark Max (`setIntakePower`); the right follows automatically.
• Beam-Break Driven Limiting (`intakeBeamStop(power)`):
    – If `disBBValue()` is FALSE (no beam / blocked depending on wiring), output = 0.0 (stop).
    – Else if `intakeBBValue()` is FALSE, output = 0.1 (slow feed to mitigate jams).
    – Else (both TRUE), output = requested `power`.
  Note: Sensor truth semantics depend on wiring (NO vs. NC). Adjust logic if inverted.

## Triggers

• `BBLockout()` → Active when `disBBValue()` is TRUE and `intakeBBValue()` is FALSE (meaning the rear is clear but front is blocked, per current semantics). Use this to gate intake commands or illuminate indicators.

## Command Factories

All commands are based on `FunctionalCommand` and require the `EndEffector` subsystem.

• `createIntakeCommand(Supplier<Double> powerSupplier)` / overload with `double`:
   – Initializes with no action, applies dynamic power each loop, stops on end/interruption, never finishes automatically.
• Presets for coral/algae:
   – `shootCoral()` → +0.5
   – `shootCoralSlow()` → +0.3
   – `manualBackFeed()` → −0.05
   – `IntakeAlgea()` → −1.0
   – `HoldAlgea()` → −0.2
   – `ShootAlgea()` → +0.2
   – `nothing()` → 0.0
• Sensor-aware intake:
   – `IntakeCoral()` → applies `intakeBeamStop(0.5)` dynamically and finishes when `!disBBValue()` (i.e., when the discharge sensor condition indicates a piece present).

## Telemetry & Dashboard

• Shuffleboard Tab: `EndEffector`
   – `BB Forward` (intake sensor), `BB Back` (discharge sensor) booleans
   – `Intake Speed` number slider (read each loop into `intakeSpeed`, currently not used to control motors directly)
• Consider wiring these entries into operator UI and/or use `intakeSpeed` as a default power for manual intake commands.

## Public API Summary

• `setIntake(double power)` / `setIntakePower(double power)` – Apply power to intake.
• `stopIntake()` – Stop motors.
• `intakeBBValue()` / `disBBValue()` – Read beam-break states.
• `BBLockout()` – Trigger based on combined sensor state.
• Command factories: `createIntakeCommand(...)`, `shootCoral()`, `shootCoralSlow()`, `manualBackFeed()`, `IntakeAlgea()`, `HoldAlgea()`, `ShootAlgea()`, `nothing()`, `IntakeCoral()`.

## Integration Notes

• Tie `IntakeCoral()` into the scoring state machine so it runs until the rear sensor registers a carried piece, then transitions to `HoldAlgea()`/`nothing()`.
• Use `BBLockout()` to prevent intake while the front path is blocked.
• Consider adding current limiting/monitoring telemetry and jam detection (e.g., sustained high current with no sensor change).

## Known Issues / Review Items

1) CTRE `TalonFX intake` is declared but never used. If you have legacy hardware or plan to drive an auxiliary roller with a TalonFX, add corresponding control methods; otherwise remove this field.
2) Sensor Semantics: `DigitalInput.get()` returns TRUE when the input line is high. Depending on wiring (NO/NC), TRUE may mean beam-unbroken or beam-broken. Validate on-robot and invert logic if necessary.
3) `intakeSpeed` from Shuffleboard is read in `periodic()` but not used elsewhere. Consider routing this to a manual intake command or as a default power.
4) Follower Inversion: `follow(LEFT_SPARKMAX, true)` inverts the right motor. Ensure physical roller directions pull inward together.
5) Add soft limits and current limits appropriate to your mechanism and game piece compression to avoid stalls.

## Safety Considerations

• Test sensor logic at low power first; confirm that pieces stop where intended and jams clear safely.
• Validate that the follower inversion is correct; opposing roll can eject or shred game pieces.
• Consider adding a timed fallback to stop motors if sensors fail to change state within a window.