# FRC Vision Subsystem Documentation (Vision.java)

## Overview

This document describes the `Vision` subsystem, a lightweight coordinator that owns two Limelight-based camera submodules (`LimelightSubsystem`): a forward-facing "bow" camera and a rear "aft" camera. While this class currently does not perform processing in `periodic()`, it centralizes configuration and exposes a single place to initialize camera extrinsics and provide vision data to the rest of the robot.

## File & Key Dependencies

• File: frc.robot.subsystems.Vision
• Submodules: `LimelightSubsystem` (this repo), wrapping `frc.robot.LimelightHelpers`
• Libraries: WPILib geometry & math (Pose3d, Matrix, VecBuilder), NetworkTables

## Architecture & Responsibilities

• Composition: Owns two `LimelightSubsystem` instances:
   – `bowLL` → constructed with camera name "limelight-bow" (front/forward camera)
   – `aftLL` → constructed with camera name "limelight-aft" (rear camera)
• Initialization: Constructor is the home for setting each camera’s pose relative to the robot (extrinsics). Two example calls (commented) are provided as a reference for `LimelightHelpers.setCameraPose_RobotSpace(...)`.
• Processing: `periodic()` is currently empty; higher-level subsystems (e.g., drivetrain pose estimator, auto-aim) should pull from `bowLL` / `aftLL` as needed.

## Camera Extrinsics (Robot-Space Pose)

Accurate camera extrinsics are critical for AprilTag-based localization:
• Use `LimelightSubsystem#setCameraPose_RobotSpace(x, y, z, yaw, pitch, roll)` per camera.
• Units: meters for translation; degrees for rotations.
• Rotation order expected by Limelight Helpers is (roll, pitch, yaw) — the subsystem maps inputs accordingly.
• Suggested process:
   1) Measure the camera lens origin relative to the robot origin (center of wheelbase on floor) for x, y, z.
   2) Measure yaw (left/right), pitch (up/down), roll (tilt).
   3) Apply and verify on-field by comparing LL pose to odometry and field references.

## Using Vision Data in Your Robot

• Robot Pose Fusion: Fetch `PoseEstimate` from each `LimelightSubsystem` via `getVisionPose()` and, when quality is sufficient, feed into your drivetrain pose estimator (e.g., `SwerveDrivePoseEstimator.addVisionMeasurement(pose, timestamp)`).
• Quality Gating: Use fields like `tagCount`, `tagSpan`, `avgTagDist`, and a custom confidence heuristic (`getConfidence()`).
• Target-relative Control: Use `getTargetInRobotSpace()` (X/Y/Z/Yaw/Pitch/Roll) for alignment and game-piece pickup/placement logic.

## Example: Fusion Gate (Pseudocode)

if (vision.bowLL.getVisionPose().tagCount >= 2 && vision.bowLL.getConfidence() > 0.6) {
    var est = vision.bowLL.getVisionPose();
    poseEstimator.addVisionMeasurement(est.pose.toPose2d(), est.timestampSeconds);
}

## Dashboard & Telemetry

Telemetry is handled in each `LimelightSubsystem` (Shuffleboard tab "Limelight"). The `Vision` class itself does not publish telemetry. Use the provided tabs to monitor tx/ty, target pose, robot pose, latency, and tag metrics for both cameras.

## Notes, TODOs, and Review Items

1) The imports `Matrix`, `VecBuilder`, `Pose3d`, and `NetworkTableInstance` are currently unused. Remove or integrate when adding fusion weighting or 3D transforms.
2) Uncomment and correct the `setCameraPose_RobotSpace` calls once you have final, measured extrinsics. Ensure the camera names match the Limelight web config.
3) Consider adding helper methods to select the "best" camera based on tag visibility (e.g., favor the one with higher tagCount and lower latency).
4) Add `periodic()` logic if you want centralized vision fusion or to republish consolidated metrics.
5) Ensure alliance-frame consistency: Limelight Blue/Red frames must match your estimator’s frame; transform when necessary.

## Safety & Testing Checklist

• Verify extrinsics: Place robot at known field marks and compare LL pose to odometry.
• Check latency: Inspect reported latency; consider rejecting measurements above a threshold (e.g., >50 ms).
• Multi-tag validation: Prefer multi-tag observations (higher tagSpan) to reduce ambiguity.
• Alignment drills: Test target-relative commands (e.g., auto-align to reef/processor) using `getTargetInRobotSpace()` from the appropriate camera.

## Public API Summary

• Fields: `bowLL`, `aftLL` — instances of `LimelightSubsystem` (front and rear cameras).
• Methods: `periodic()` — currently empty placeholder for future aggregation logic.