# FRC Wrist Subsystem Documentation (Wristsp.java)

## Overview

This document describes the `Wristsp` subsystem, a WPILib `SubsystemBase` that controls a single REV Spark Max–driven wrist with an integrated absolute encoder. The subsystem supports manual control, PID-based position control, homing using a limit switch, soft limit logic, preset positions for common tasks, and Shuffleboard telemetry for tuning and driver feedback.

## Hardware & I/O

• Motor: 1× REV Spark Max brushless controller (`LEFT_SPARKMAX` from `WristConstants`).
• Encoder: Spark Absolute Encoder obtained from the Spark Max (`getAbsoluteEncoder()`), inverted in config.
• Limit Switch: Digital input on DIO 4, used as a home/zero reference for `homeCommand()`.
• Dashboard: Shuffleboard tab "Wrist" with live position and setpoint entries; SmartDashboard prints for debug.

## Motor & Sensor Configuration

• Idle Mode: Brake — holds position when unpowered.
• Smart Current Limit: `WristConstants.WRIST_CURRENT_LIMITS`.
• Encoder: Absolute encoder with position/velocity conversion factors set to 1.0, encoder inverted.
• Persistence: Configuration applied with `ResetMode.kResetSafeParameters` and `PersistMode.kPersistParameters`.

## Control Model

• PID: `PIDController`(kP=3.4, kI=0, kD=0) with tolerance 0.0075. The controller drives the wrist toward a setpoint.
• Feedforward-like gravity compensation: In `setWristMotor()`, an offset based on `sin(position * 2π)` is added to the requested power. This provides simple gravity compensation that varies with angle. The magnitude is ~0.02–0.02125 depending on wrist position.
• Output limiting: `wristLimit()` enforces motion bounds depending on absolute encoder position ranges.

## Motion Limits & Safety Windows

The `wristLimit(double speed)` method constrains output based on encoder position:
• If 0.315 < position < 0.7 and commanded speed > 0 (moving further into that zone), output → 0.
• If position ≥ 0.7 OR position < 0.109 and commanded speed < 0, output → 0.
These windows prevent over‑travel into mechanical hard stops or collision zones. Tune thresholds to your build.

## Public API Summary

• `double getWristPosition()` — current absolute position (encoder units; 0–1 rotations if default scaling).
• `void setWristPID(double setPoint)` — updates PID setpoint.
• `void executeWristPID()` — runs PID loop and applies motor output (with limit & compensation).
• `boolean wristAtSetpoint()` — PID within tolerance.
• `Trigger wristLimiter()` — placeholder trigger (returns true); intended for gating folds.
• `Trigger wristIntake()` — true when position ≥ 0.42 (useful for enabling intake actions).

## Command Factories

All commands are `FunctionalCommand`s that require the `Wristsp` subsystem.

• `WristPIDCommandDefault(BooleanSupplier canFold)` — continuous PID position hold.
• `ManualWrist(DoubleSupplier wristJoystick, BooleanSupplier canFold)` — manual control (±0.3 scale); on end, zeros output and snaps setpoint to current position.
• `homeCommand()` — drives slowly negative (−0.1) until the DIO limit switch is triggered; then stops and sets PID setpoint to current position.
• `raiseWrist()` — drives positive at +0.3 continuously (no auto‑finish).
• `wristCommandFactory(BooleanSupplier canFold, double position)` — general position move command; ends when `wristAtSetpoint()`.
• Presets (all via `wristCommandFactory`):
    – `WristSafety(…)` → 0.25
    – `WristL0(…)` → 0.23
    – `WristAlgeaIntake(…)` → 0.17
    – `WristL1(…)` → 0.25
    – `WristL2(…)` → 0.179
    – `WristL3(…)` → 0.167
    – `WristL4(…)` → 0.0887
    – `WristA1(…)` → 0.20
    – `WristA2(…)` → 0.210
    – `WristBarge(…)` → 0.289
    – `WristProcessor(…)` → 0.20
    – `WristClimber(…)` → 0.40
• `ExitState(BooleanSupplier)` — hold current position.

## Telemetry & Shuffleboard

Tab: "Wrist"
• Entries: `WristValue` (current position), `WristSetpoint` (PID setpoint).
• SmartDashboard: `wrist limit` (DIO), plus debug numbers for axis/speed within `setWristMotor()`.

## Integration Notes & Best Practices

• Units: Absolute encoder values are used directly; set conversion factors if you want radians or degrees.
• Gravity Compensation: Verify the sine‑based offset against your mechanism’s center of mass; consider moving to a kS/kG/kV feedforward model for precision.
• Safety: Validate `wristLimit()` thresholds on‑robot. Ensure `homeCommand()` direction matches switch orientation and won’t crash.
• Triggers: Replace the placeholder `wristLimiter()` with a real condition (e.g., disallow folding unless elevator is above threshold).

## Troubleshooting & Tuning

• If the wrist buzzes while holding, reduce kP or adjust the gravity compensation magnitude.
• If it sags when horizontal, increase the base compensation or add static feedforward.
• If movement is blocked unexpectedly, check `wristLimit()` ranges and the encoder inversion.
• For overshoot/oscillation, add small kD or lower kP.